# GE90-115B Modeling Basis and pyCycle Map Conversion

Interim Review v0.1

Purpose. This review copy consolidates the public GE90-115B data, first-order calculations, and the current NASA/pyCycle conversion plan for a high-fidelity real-time engine model. Values are tagged as Published, Calculated, or Estimated so that later tuning does not obscure their origin. The current map section contains the component targets and scaling equations; point-by-point converted map arrays remain a separate implementation step once the exact pyCycle source arrays are loaded into the model-development environment.

# 1. Modeling Approach

The GE90 model will use the existing TurboFan architecture: flight condition and inlet total-state calculation, map-based fan/LPC/HPC aerodynamics, combustor energy balance, HPT/LPT work extraction, independent HP and LP shaft dynamics, and separate core and bypass nozzles. NASA/pyCycle maps provide the initial aerodynamic surfaces. The GE90 conversion is performed by matching corrected flow, pressure ratio, efficiency and corrected speed at selected design points while preserving the underlying map topology. Transient response is subsequently governed by shaft work imbalance, rotor inertia and numerical integration rather than by separate transient lookup schedules.

# 2. Published GE90-115B Anchors

| Parameter | Initial value | Status |
| --- | --- | --- |
| Architecture | 2 spool; 1-stage fan, 4-stage booster/LPC, 9-stage HPC, 2-stage HPT, 6-stage LPT | Published |
| Fan diameter | 128 in / 3.251 m | Published |
| Fan blades | 22 composite blades | Published |
| Sea-level takeoff thrust | 115,540 lbf / 513.95 kN | Published |
| Maximum continuous thrust | 110,000 lbf / 489.3 kN | Published |
| Overall pressure ratio | ~42 to 42.24 | Published |
| Fan pressure ratio | ~1.50 | NASA reference |
| HPC pressure ratio | ~19:1 | Published engineering reference |
| Nominal bypass ratio | ~9 | Published/NASA reference |
| Sea-level-static bypass ratio | ~7.1 | NASA database value |
| 100% N1 reference | 2,355 rpm | Published/certification |
| Maximum N1 | 2,602 rpm / 110.5% | Published/certification |
| 100% N2 reference | 9,332 rpm | Published/certification |
| Maximum N2 | 11,292 rpm / 121% | Published/certification |
| Takeoff T49 limit | 1,090 degC | Published/certification |
| Maximum-continuous T49 limit | 1,050 degC | Published/certification |
| Ground-start T49 limit | 750 degC | Published/certification |
| Fuel lower heating value | 42.798 MJ/kg rating basis | Published/certification |
| NASA modeled T41 | ~1,786 K | NASA non-proprietary NPSS result |
| Takeoff-roll total airflow | ~1,350 kg/s | Published academic analysis |
| Cruise total airflow | ~576 kg/s | Published academic analysis |
| Cruise TSFC anchor | ~0.550 lb/(lbf hr), M0.80 / 35,000 ft | NASA database value |

# 3. First-Order Derived Quantities

| Derived quantity | Result | Basis |
| --- | --- | --- |
| Booster/LPC pressure ratio | 42 / (1.50 x 19) = 1.474 | Cycle closure from OPR |
| Average booster stage PR | 1.474^(1/4) = 1.102 | Four-stage geometric average |
| Average HPC stage PR | 19^(1/9) = 1.387 | Nine-stage geometric average |
| Gross fan disk area | pi x 3.251^2 / 4 = 8.30 m^2 | Fan diameter |
| Fan tip speed at 100% N1 | pi x 3.251 x 2355 / 60 = ~401 m/s | Reference N1 |
| Fan tip speed at max N1 | pi x 3.251 x 2602 / 60 = ~443 m/s | Maximum N1 |
| SLS core flow using BPR 7.1 | 1350 / (1 + 7.1) = 166.7 kg/s | Initial splitter target |
| SLS bypass flow using BPR 7.1 | 1350 - 166.7 = 1183.3 kg/s | Initial splitter target |
| Takeoff FAR, before cooling/bleed correction | 4.69 / 166.7 = 0.0281 | Uses ICAO takeoff fuel-flow point |
| Fuel chemical heat input at takeoff | 4.69 x 42.798 x 0.99 = ~199 MW | Assumes 99% burner efficiency |

# 4. Provisional Component Targets

| Component | Base map | Design-flow target | PR / expansion target | Efficiency target | Primary constraint |
| --- | --- | --- | --- | --- | --- |
| Fan | NASA/pyCycle FanMap | ~1350 kg/s total SLS | 1.50 | 0.89-0.90 initial | LP shaft |
| LPC / booster | NASA/pyCycle LPCMap | ~166.7 kg/s before bleeds | 1.474 | 0.90-0.92 initial | LP shaft |
| HPC | NASA/pyCycle HPCMap | slightly below LPC after cooling/bleed splits | 19.0 | 0.88-0.90 initial | HP shaft |
| HPT | NASA/pyCycle HPTMap | core + fuel + returning cooling flow | ~4.04 expansion, provisional | ~0.90 initial | HP shaft work balance |
| LPT | NASA/pyCycle LPTMap | post-HPT core stream | ~3.34 expansion, provisional | ~0.90 initial | LP shaft work balance |

# 5. pyCycle Map Scaling

pyCycle computes four map scalars at the design point:

s_Nc = Nc / Nc_map

s_Wc = Wc / Wc_map

s_PR = (PR - 1) / (PR_map - 1)

s_eff = eff / eff_map

The off-design map then applies the inverse transformation to every interpolated map point: PR = 1 + (PR_map - 1)s_PR, Wc = Wc_map s_Wc, efficiency = efficiency_map s_eff, and Nc = Nc_map s_Nc. This lets the GE90 model retain the NASA map curvature, speed-line structure, surge relationship and efficiency islands while moving the design point to the GE90 operating region.

# 6. Cycle-Level Comparison with the pyCycle HBTF Example

| Quantity | pyCycle HBTF design setting | GE90 provisional target | Relative factor / note |
| --- | --- | --- | --- |
| Fan PR | 1.685 | 1.50 | (1.50-1)/(1.685-1) = 0.730 |
| Fan efficiency | 0.8948 | 0.895 initial | ~1.000 |
| LPC PR | 1.935 | 1.474 | (1.474-1)/(1.935-1) = 0.507 |
| LPC efficiency | 0.9243 | 0.91 initial | 0.985 |
| HPC PR | 9.369 | 19.0 | (19-1)/(9.369-1) = 2.151 |
| HPC efficiency | 0.8707 | 0.89 initial | 1.022 |
| LP mechanical speed | 4666.1 rpm | 2355 rpm at 100% N1 | 0.505 reference ratio |
| HP mechanical speed | 14705.7 rpm | 9332 rpm at 100% N2 | 0.635 reference ratio |

Important: the table above compares cycle-level design settings, not the internal raw map design coordinates. The final s_Wc and s_Nc values must be computed from the actual FanMap/LPCMap/HPCMap/HPTMap/LPTMap design-point values after those arrays are imported. The pressure-ratio and efficiency relationships shown here are still useful as first-order checks.

# 7. Combustor and Fuel Anchors

| Condition | Fuel flow | Use |
| --- | --- | --- |
| Idle / 7% ICAO | 0.38 kg/s | Validation point |
| Approach / 30% ICAO | 1.13 kg/s | Validation point |
| Climb-out / 85% ICAO | 3.67 kg/s | Validation point |
| Takeoff / 100% ICAO | 4.69 kg/s | Validation point |

FAR remains a calculated result of burner air flow and commanded fuel flow. Cooling, bleed and customer extraction must therefore be resolved before final burner FAR and turbine-flow values are established. T41 is treated as a cycle result constrained by fuel flow and burner energy balance, while T49 is the certification/validation temperature downstream of the HPT.

# 8. Items Still Requiring Engineering Identification

Exact point-by-point pyCycle base-map arrays and their default design coordinates for the five selected maps.

Internal cooling-flow fractions and their return locations.

Variable stator and compressor bleed schedules.

Core and bypass nozzle effective areas and discharge coefficients.

HP and LP rotating inertias for transient matching.

Accessory extraction schedule versus operating condition.

Acceleration/deceleration fuel limits, EGT limiting, and other FADEC schedules.

Final validation against multiple thrust, fuel-flow, altitude, Mach and spool-speed operating points.

# 9. Current Review Status

Ready for review now: engine architecture, public GE90 anchor data, first-order derived values, provisional component design targets, pyCycle scaling equations, and a master workbook that exposes all assumptions and formulas.

Not yet represented as a final GE90 map deck: the full point-by-point scaled Fan/LPC/HPC/HPT/LPT arrays. The workbook is deliberately structured so those raw pyCycle arrays can be inserted without changing the GE90 target or calculation sheets.

# Sources Used in This Interim Review

OpenMDAO pyCycle repository and high-bypass turbofan example: https://github.com/OpenMDAO/pyCycle

pyCycle high-bypass turbofan example: https://github.com/OpenMDAO/pyCycle/blob/master/example_cycles/high_bypass_turbofan.py

pyCycle compressor-map scaling implementation: https://github.com/OpenMDAO/pyCycle/blob/master/pycycle/elements/compressor_map.py

NASA Technical Reports Server: https://ntrs.nasa.gov/

GE Aerospace GE90 material: https://www.geaerospace.com/

EASA GE90 type-certificate data: https://www.easa.europa.eu/

Stanford aircraft and rocket propulsion course material: https://web.stanford.edu/