TurboFanModel — Real-Time Physics-Based Turbofan
Modeling Framework

TurboFanModel is a real-time, system-level turbofan modeling framework for propulsion simulation, controls research, hardware-in-the-loop applications, engine-health studies, and aircraft integration. A complete engine definition can be constructed from public performance data, geometry, NASA/pyCycle component maps, and documented engineering analysis.

Technical approach

The engine is solved as a coupled aero-thermodynamic and mechanical system. Continuously interpolated fan, compressor, and turbine maps supply component performance while Brayton-cycle relationships and conservation equations determine pressures, temperatures, flows, shaft work, fuel requirements, and thrust. HP and LP spool response follows differential shaft work/torque equations with rotor inertia and is integrated in real time using numerical methods such as fourth-order Runge-Kutta. Inlet, ambient, Mach, bleed, accessory load, nozzle, and control effects are part of the same dynamic solution.

Engine model generation and traceability

An engine definition begins with the best available published geometry and performance data. Missing quantities are calculated where physical relationships permit and otherwise estimated from established engineering methods. NASA/pyCycle generic maps provide the aerodynamic basis and are scaled or reshaped to satisfy engine-specific design points and off-design constraints. Each defining value is identified as published, calculated, estimated, or tuned so its technical basis remains visible.

Current demonstration — GE90-115B

The first full demonstration is a GE90-115B model. Public GE, certification, NASA, and technical-literature data are being assembled into a master parameter set and used to generate GE90-specific Fan, LPC, HPC, HPT, and LPT maps from NASA/pyCycle data. Validation will progress from static and transient operation in TurboFanLab to altitude/Mach testing and full-aircraft operation with a Boeing 777 model using available flight-simulation programs such as FlightGear/JSBSim. The objective is to validate the modeling process across an operating envelope rather than at a single design point.

Integration and research use

TurboFanModel is designed as a self-contained real-time engine module with defined external inputs and outputs. It can be integrated with available flight-simulation programs and research environments such as FlightGear/JSBSim with minimal host-side disruption, and can also interface with external controls, instrumentation, actual hardware, and hardware-in-the-loop systems. The same architecture can support propulsion-control studies, hybrid-electric shaft-power extraction, engine-health and degradation research, aircraft/propulsion integration, and additional turbofan engine definitions.